# Grasshopper Beginner's Walkthrough — Orthotic Insole

# Grasshopper Beginner's Walkthrough — Orthotic Insole

Zero Grasshopper experience required. Follow each step in order. Estimated

time the first time: 20–30 minutes. After that, opening the file and moving

sliders takes seconds.

────────────────────────────────────────

## 0. What is Grasshopper?

Grasshopper is a visual programming canvas that lives inside Rhino. You

drop "components" (boxes) and connect their outputs to other components'

inputs with wires. When you change a value, everything downstream rebuilds

automatically. That live-updating is exactly what we want for the insole.

You will not write any code. You'll paste Python code that I've already

written into a special "Python 3 Script" component.

────────────────────────────────────────

## 1. Open Grasshopper

Open Rhino 8.

In the Rhino command line, type Grasshopper and press Enter.

A second window opens — that's the Grasshopper canvas. Keep both windows

visible side by side.

The Grasshopper canvas has:

A ribbon at the top with tabs (Params, Maths, Sets, Vector, Curve,

Surface, Mesh, Intersect, Transform, Display, Script).

A blank canvas below.

────────────────────────────────────────

## 2. Reference your shoe last from Rhino

The shoe last lives in your Rhino document. We need to "reference" it into

Grasshopper.

In Rhino, open the file containing your shoe last (the polysurface or

SubD you've been working with).

In Grasshopper, go to the Params tab → Geometry group → drag a

Brep parameter onto the canvas. (It's an empty grey hexagon labelled

Brep.)

Right-click the Brep parameter → Set one Brep.

Grasshopper hides itself and Rhino activates. Click your shoe last.

The Brep parameter on the canvas turns from grey to having a small

preview indicator. Done — the last is now an input.

If your last is a **SubD**, drop a `SubD` parameter from Params → Geometry

instead, then add a **SubD to Brep** component (Surface → SubD → SubD to

Brep) between it and the script. Convert once, you don't have to do it

every session.

────────────────────────────────────────

## 3. Drop the Python script component

Double-click on the empty canvas anywhere. A search box appears.

Type python 3 and pick Python 3 Script (green/cyan icon, the one

that says "CPython 3"). Press Enter.

A small component appears with default inputs x and y and output a.

This is the component we'll fill with the insole code.

────────────────────────────────────────

## 4. Configure the script component's inputs

The all-in-one script needs 11 inputs. We'll add them one by one.

### 4a. Open the script editor

Double-click the Python 3 Script component. The script editor opens.

### 4b. Replace the default code

Select all the placeholder code in the editor (Ctrl+A) and delete it.

Open Grasshopper/components/00_orthotic_insole_all_in_one.py from this

repo in any text editor.

Copy its entire contents and paste into the GH script editor.

Click Test (bottom of the editor) — you'll see errors about undefined

names like Last, PerimeterOffset. That's expected — we haven't created

the inputs yet. Click OK to close any error popup.

Close the editor (the X). The component now lives on the canvas with the

pasted code.

### 4c. Add the inputs

The script component starts with two inputs (x, y). We need 11. Here's

how to add and rename them:

To rename an input:

Right-click on the input name (e.g. x on the left side of the

component) → choose "x" at the top → a text field appears → type the

new name → press Enter.

To add more inputs:

Zoom in on the component until you see small + and − icons next to

the inputs. Click + to add an input.

(Alternative: right-click the component header → Add Input.)

Rename and add until your component has these 11 inputs, top to bottom:

| Input name | Type hint | Access |
| --- | --- | --- |
| `Last` | Brep | Item |
| `PerimeterOffset` | float / Number | Item |
| `ToeExtension` | float / Number | Item |
| `HeelExtension` | float / Number | Item |
| `CoverThk` | float / Number | Item |
| `ShellThk` | float / Number | Item |
| `BaseThk` | float / Number | Item |
| `OverrideOutline` | Curve | Item |
| `OverrideBottom` | Curve | Item |
| `MaxEdge` | float / Number | Item |
| `MinEdge` | float / Number | Item |

(RebuildCount is optional — add it if you want to control curve smoothness;

otherwise default 20.)

To set a type hint:

Right-click the input name → Type hint → pick from the list.

Pick exactly one of these for each input:

| Input | Type hint |
| --- | --- |
| `Last` | **Brep** |
| `OverrideOutline` | **Curve** |
| `OverrideBottom` | **Curve** |
| `PerimeterOffset` | **float** |
| `ToeExtension` | **float** |
| `HeelExtension` | **float** |
| `CoverThk` | **float** |
| `ShellThk` | **float** |
| `BaseThk` | **float** |
| `MaxEdge` | **float** |
| `MinEdge` | **float** |

If your GH version shows Number instead of float, pick that — same thing.

To set access:

Right-click the input name → Access → Item Access.

### 4d. Add the outputs

The default output is a. We need 8 outputs. Same procedure on the right

side:

Output name

`SubD`

`Mesh`

`Footprint`

`Outline`

`TopEdge`

`BottomEdge`

`Axis`

`ToeDir`

Outputs don't need type hints — Grasshopper figures them out.

────────────────────────────────────────

## 5. Wire the shoe last in

Drag a wire from the Brep parameter (right-side circle) on the

referenced last to the Last input (left-side circle) on the script

component.

The script component now has the last as input. If you hover over the

component, you'll see "Last: 1 Brep" in the tooltip.

If the script border is red, hover over it — Grasshopper will say what's

missing. Most likely: "PerimeterOffset is None" because we haven't wired

sliders yet. Continue.

────────────────────────────────────────

## 6. Add number sliders

You need 6 sliders for the everyday parameters. Here's the fast way:

Double-click empty canvas → type 2..0..10 and press Enter.

- This is shorthand: default=2, min=0, max=10. A slider drops onto

the canvas pre-set.

Right-click the slider → Edit → change Name to

PerimeterOffset (helpful when you have many sliders).

Drag a wire from the slider's right side to PerimeterOffset on the

script.

Repeat for each slider:

| Slider name | Shorthand to type | Default |
| --- | --- | --- |
| PerimeterOffset | `2..0..10` | 2.0 mm |
| ToeExtension | `0..-10..30` | 0.0 mm |
| HeelExtension | `0..-10..30` | 0.0 mm |
| CoverThk | `2..0.5..10` | 2.0 mm |
| ShellThk | `3..0.5..15` | 3.0 mm |
| BaseThk | `5..0.5..20` | 5.0 mm |

For MaxEdge and MinEdge, leave them unwired — the script falls back to

defaults (3.0 and 1.0). Or add sliders if you want finer control.

────────────────────────────────────────

## 7. Preview the SubD

The SubD output on the right of the script is what you want to see.

Right-click in empty canvas → search SubD → drop a SubD parameter.

Wire SubD output → the SubD parameter input.

The SubD parameter has a small icon showing the geometry preview is on

(a grey/coloured bulb). If it's off, right-click the parameter → Preview.

Look at your Rhino viewport — the insole should now be visible as a blue

SubD shape sitting on top of your shoe last.

If you don't see anything in Rhino:

Check the script component isn't red. Hover for the error message.

Make sure the Brep parameter is correctly referenced (right-click → "Set

one Brep" again if needed).

Ensure your shoe last is oriented sole-down (sole facing −Z).

────────────────────────────────────────

## 8. Move sliders — this is the live updating

Drag the PerimeterOffset slider. The insole rebuilds every time you let

go (or in real time if you hold and drag — depends on your GH solver mode).

Try:

PerimeterOffset from 2 to 5 — the insole shrinks inward.

ToeExtension to 10 — the toe end gets longer.

BaseThk to 10 — the insole gets thicker.

That's the whole live-updating workflow. Move sliders, watch geometry update.

────────────────────────────────────────

## 9. View the outline curves

To see the editable top and bottom edges:

Drop two Curve parameters from Params → Geometry.

Wire TopEdge → first Curve param, BottomEdge → second.

Both curves now preview in Rhino as orange/red lines hugging the insole.

────────────────────────────────────────

## 10. Edit control points (the "drag points to reshape" workflow)

This is the big advantage over the .rhi plugin.

Right-click the TopEdge Curve parameter → Bake.

- A dialog asks for layer / attributes — accept defaults, click OK.

- The curve is now a real Rhino object in your document.

In Rhino, select that curve and turn on its grips: type PointsOn

(or press F10).

Drag the control points wherever you want. Each drag moves the curve.

Back in Grasshopper, drop a Curve parameter from Params →

Geometry. Right-click → Set one Curve → pick the edited curve in

Rhino.

Wire that Curve param into the script's OverrideOutline input.

The insole rebuilds using your dragged shape. Sliders still work, but

PerimeterOffset / ToeExtension / HeelExtension are now ignored —

your manual outline takes over.

Same procedure for BottomEdge → OverrideBottom if you want to sculpt

the underside.

To go back to auto-generated outline: right-click the Curve param feeding

OverrideOutline → Disconnect the wire. The script will generate the

outline from sliders again.

────────────────────────────────────────

## 11. Save the definition

In Grasshopper: File → Save As → name it OrthoticInsole.gh.

Next time you want to use it: open Rhino with your shoe last, then in

Grasshopper File → Open → pick the .gh file. The slider values and

Last reference are saved with the file.

────────────────────────────────────────

## 12. Bake the final insole to your Rhino document

When you're happy with the shape:

Right-click the script component's SubD output → Bake.

Pick a layer, click OK.

The SubD is now a permanent Rhino object you can export, mesh, 3D print,

or process further.

Same for TopEdge, BottomEdge, Footprint, Outline if you want

them as documented curves.

────────────────────────────────────────

## Common problems

"The script component is red"

Hover over it. Read the error message at the top of the popup.

Most common cause: a required input is unwired or the wrong type. Check

your wires.

"My insole is upside down / wrong size"

The shoe last needs to have the sole facing −Z (downward). In Rhino,

rotate it before referencing.

"Insole is missing — only outline shows up"

Probably Last is not wired or the Brep param has lost its reference.

Right-click the Brep param → "Set one Brep" again.

"Sliders don't update the geometry live"

Check Solver menu (top of GH window) → make sure it's not in Disabled

state (the icon should NOT have a red slash through it).

"Component is yellow with a warning"

Yellow = warning, not error. Hover for details. Often it's "no curves

found in section X" — usually safe to ignore if the SubD output still

comes through.

────────────────────────────────────────

## What each output does

SubD — smooth subdivision surface, the deliverable. Bake this.

Mesh — the underlying mesh before SubD conversion. Useful if SubD

fails (then SubD will be empty and you fall back to baking the mesh).

Footprint — flat XY outline traced from the shoe last sections.

Outline — the offset insole perimeter (still flat XY).

TopEdge — 3D NURBS curve following the sole-conforming top.

BottomEdge — 3D NURBS curve along the flat underside.

Axis — "X" or "Y", just informational.

ToeDir — +1 or -1, just informational.

────────────────────────────────────────

## Tip: snapshot your slider settings

Once you've got values that work for a particular last:

Note down the slider values, OR

Save the .gh file with that last as a custom file

(Mens_Size_10_Insole.gh).

Each .gh file is just a definition + slider state. You can keep many of

them per shoe last / customer.